SKB Ethernet Test Result v0.1        2019/05/03

- Introduction.

This document specifies the details to validate DUT Ethernet performance. This test will use Iperf3.1.3/Ethtool Test tool and Control NB.

1.1. Auto Negotiation Test

- Test Environment

- Configure the test setup as shown in block diagram below

10/100/1000BaseT OTT Box

10/100/1000BaseT NoteBook

Link

Figure 1:  Auto Negotiation Test

Purpose: Verify the DUT responds to management by transmitting the

appropriate link signaling.

The performance specified in following paragraphs shall be guaranteed under the following conditions:

| Test Items # | Description | Test Status |
| --- | --- | --- |
| 1 | 1000BaseT – Full duplex | PASS |
| 2 | 1000BaseT – Half duplex | PASS |
| 3 | 100BaseT – Full duplex | PASS |
| 4 | 100BaseT - Half duplex | PASS |
| 5 | 10BaseT – Full duplex | PASS |
| 6 | 10BaseT - Half duplex | PASS |

1.2. CRC Error Test

- Test Environment

- Configure the test setup as shown in block diagram below

10/100/1000BaseT OTT Box

Notebook

Cisco 891FW

Figure 2:  CRC Error Test

The performance specified in following paragraphs shall be guaranteed under the following conditions:

| Test Items # | Description | Test Status |
| --- | --- | --- |
| 1 | 1000BaseT – Full duplex | Fail |
| 2 | 100BaseT – Full duplex | PASS |
| 3 | 10BaseT – Full duplex | PASS |

Purpose: Verify the DUT can detect accidental changes to raw data.

- 1000BaseT :

- NB:

To run this test , we will open our command processor and input the following information.

“ iperf3.exe –c 192.168.1.2 –t 3600 –w 90k”

This should look like the following :

- DUT:

This command will run a iperf test two hours.

“ iperf3.exe –s &”

The test results are as follows :

- 100MBaseT :

- NB:

To run this test , we will open our command processor and input the following information.

“ iperf3.exe –c 192.168.1.2 –t 3600 –w 20k”

The test results are as follows :

- 10MBaseT :

- NB:

To run this test , we will open our command processor and input the following information.

“ iperf3.exe –c 192.168.1.2 –t 3600 –w 10k”

The test results are as follows :