2. BMT 의뢰서

2.1. HW 사양

| Chipset | Amlogic S905X2 |
| --- | --- |
| Flash | eMMC 8GB |
| DRAM | DDR4 3GB |
| OS | Android TV "Pie" ver |
| Video Decoding | H.264 BP/MP/HP@L4.x, H.265 HEVC MP/MP10@L5.1, VP9, MPEG4, MPEG2, MJPEG |
| Audio Decoding | AAC, HE-AAC, Dolby Digital Plus MPEG-1 Audio Layer1, 2, 3 PCM(LPCM / ADPCM) |
| Bluetooth | BT4.2 |
| Front | 1 X LED (Red/Green/Blue) 1 X IR |
| TOP | 2 X KEY (CH+/-) |
| Rear | 1 X HDMI 2.0 1 X 10/100/1000 Base-T Ethernet 1 X DC 5V/2A IN 1 X USB 3.0 1 X MIC Jack |
| Power Supply | 5V/2A |

2.1. 1. Block diagram

2.1. 2. PCB Assay

a. Set-Top Main Board assy: TOP Side

b. Set-Top Main Board assy: BOTTOM Side

2.1.3. 제품 디자인

(최종 렌더링 이미지는 10/19/금 제출 예정.)

- Front

- Rear

- Left

- Right

- Top

- Bottom

2.1.4. Accessories

-HDMI cable *1

-Cat. 5e LAN cable *1

-User Manual *1

-5V/2A Adapter *1

-IR/BT RCU *1

2.1.5. Main Parts List

| Item | PART DESCRIPTION | Quantity |
| --- | --- | --- |
| 1 | Audio ADC: IC DSP 3.6V SMD QFN24 8~96KHz GP/HF | 1 |
| 2 | BT: IC MPU 3.3V SMD FCBGA49 2.4GHz GP/HF | 1 |
| 3 | GIGA Ethernet PHY: IC PER 1.5/1.8/2.5/3.3V SMD QFN40 GP ETHERNET TRANSCEIVER | 1 |
| 4 | Main SOC: IC PER 1.8/3.3V SMD FCBGA393 GP ARM | 1 |
| 5 | DDR4 8Gb: IC DDR4 SDRAM 8Gb 1.2V 0.625nS SMD FBGA96 GP | 2 |
| 6 | EMMC 8GB: IC eMMC 8GB 2.7~3.6V 5nS SMD FBGA153 GP | 1 |
| 7 | IR: IR RECEIVER 38KHz SMD SMD4 GP | 1 |
| 8 | Microphone Input: CONN AUDIO JACK 1*1PORT D=3.6/7.0mm DIP F RT GP | 1 |
| 9 | USB 3.0: CONN USB 3.0 A 1*1PORT 2.0mm DIP F RT GP/HF | 1 |
| 10 | HDMI: CONN HDMI A 19PIN 1*1PORT 0.5mm SMD MALE RT GP | 1 |
| 11 | RJ-45: CONN RJ45 8P8C 1u"GOLD 1*1PORT W/LED Y/G 2.04mm DIP F RT GP BLACK | 1 |
| 12 | DDR4 4Gb: IC DDR4 SDRAM 4Gb 1.2V 0.625nS SMD FBGA96 GP | 2 |
| 13 | DC Jack: CONN PWR JACK 1*1PORT D=2.0/6.5mm DIP FEMALE RT GP 5V 2A | 1 |