BTV Framework - HAL

Git repository for BTF HAL

| HAL interface declaration | ssh://git@bitbucket.skbroadband.com:7999/stbbcf/hal-interface.git |
| --- | --- |
| HAL testsuite | ssh://git@bitbucket.skbroadband.com:7999/stbbcf/hal-testsuite.git |
| HAL implementation - dummy | ssh://git@bitbucket.skbroadband.com:7999/stbbcf/hal-impl_dummy.git |

BTF HAL

AV interface : https://bitbucket.skbroadband.com/projects/STBBCF/repos/hal-interface/browse/btv_hal.h?at=refs%2Fheads%2FBTF_API_LEVEL_2

Downloading Source Code

SKB Bitbucket

https://bitbucket.skbroadband.com/projects/STBDEV_BFX-AT100-BTF

Initializing a Repo client

repo init -u ssh://git@bitbucket.skbroadband.com:7999/stbdev_bfx-at100-btf/manifest.git -b BFX-AT100_5.1.3_BTF

Branch for BTF development : BFX-AT100_5.1.3_BTF

Derived from the manifest file:  BFX-AT100_5.1.3_2020-0130-154149_r110.xml

BTF_API_LEVEL_2

| Return | Funtion | Description |
| --- | --- | --- |
| Return | Funtion | Description |
| Int | BTF_HAL_Initialize | Initialize BTF HAL. Reserve resource in advance. |
| Int | BTF_HAL_Terminate | Terminate BTF HAL. Release resource. |
| int | VCOMPO_SetVideoPlaneRect | set rectangle of video plane |
| int | DISPLAY_HDMI_SetEventListener | Existing event listener receives HPD only. This listener receives all HDMI events. Includes HDCP. Receive all middleware and process only what is needed in middleware. All events will be saved as a file, like a timestamp, and sent to the server.. |
| int | DISPLAY_CEC_SetEventListener | Receive all CEC messages as well as power state. Receive all middleware and process only what is needed in middleware. All events will be saved as a file, like a timestamp, and sent to the server. |
| int | DEVICE_GetSOCUniqueID | Unique id of chipset |

1. SOC Media API

| Return | Funtion | Description |
| --- | --- | --- |
| Return | Funtion | Description |
| Int | AVP_Create | creates player and does initial works for this player |
| Int | AVP_Destroy | destroys the player |
| Int | AVP_Start | starts player |
| Int | AVP_Pause | pauses the player |
| Int | AVP_Resume | resumes the Player |
| Int | AVP_Stop | stop player |
| Int | AVP_InjectTS | feeds some TS data used for playing in input buffer |
| int | AVP_InjectCommand | performs the corresponding operation according to the command given when feeds some TS data used for playing in input_buffer |
| Int | AVP_GetVideoPTS | gets the current displaying frame's pts |
| Int | AVP_FlushTS | Drop on/off Demux/decoder Drops all previously fed TS data from internal buffer which is not yet processed |
| Int | AVP_SetPlaySpeed | sets the speed of play |
| Int | AVP_SetVideoPlaneSize | sets the window area by specific size |
| int | AVP_SetSurface | sets android native window |
| Int | AVP_SetAVOffset | sets the video delay by specific millisecond |
| int | AVP_SetVideoMute | sets the player with specific handle video mute. |
| Int | AVP_SetAudioMute | sets the player with specific handle audio mute. (NOT system wide mute, just for player) |
| Int | AVP_GetAudioMute | gets a specific player audio mute status |
| Int | AVP_SetAudioStream | changes audio pid, audio codec type, then play back the audio |
| Int | AVP_SetSectionFilter | opens and sets sectionFilter by specific pid |
| Int | AVP_ClearSectionFilter | stops the sectionFilter |
| int | AVP_SetCommand | performs the corresponding operation according to the command given. |

1.1  SOC Media Callback API

| callback | function | descriptor |
| --- | --- | --- |
| callback | function | descriptor |
| AVP_MediaCallback | typedef void (*AVP_MediaCallback)(AVP_MediaCallbackEvent event, void * data, uint32_t data_size, void * callback_user_param) | Funtiont pointer for event callback |
| AVP_SectionFilterCallback | typedef void (*AVP_SectionFilterCallback)(const unsigned char *section, const unsigned int length, void *section_user_param) | Function pointer for section callback |

2. SOC AudioMixer API

| Return | Function | Description |
| --- | --- | --- |
| Return | Function | Description |
| Int | AMIXER_Create | creates audioMixer module and does initial works for this audioMixer module |
| Int | AMIXER_Destroy | destroys the audioMixer module |
| Int | AMIXER_SetMasterAudioMute | sets system wide audio mute. |
| Int | AMIXER_SetAudioOutputMode | sets audio output mode(AVP_AUDIO_PASSTHROUGH or AVP_AUDIO_PCM) |
| Int | AMIXER_GetAudioOutputMode | gets audio output mode(AVP_AUDIO_PASSTHROUGH or AVP_AUDIO_PCM) |

3. SOC VideoCompositor API

| Return | Function | Description |
| --- | --- | --- |
| Return | Function | Description |
| Int | VCOMPO_Create | creates video component and does initial works for this video component |
| Int | VCOMPO_Destroy | destroys the video component |
| Int | VCOMPO_SetZorder | sets the video zorder |
| int | VCOMPO_SetVideoPlaneRect | set rectangle of video plane |
| Int | VCOMPO_SetScaleRatioMode | sets the video scale ratio by specific mode (AVP_ASPECT_RATIO_ORIGINAL_SCREEN or AVP_ASPECT_RATIO_FULL_SCREEN) |
| Int | VCOMPO_SetVideoBrightness | Set Graphic Brightness Value |
| Int | VCOMPO_GetVideoBrightness | Get Graphic Brightness Value |
| Int | VCOMPO_SetVideoContrast | Set Graphic Contrast Value |
| Int | VCOMPO_GetVideoContrast | Get Graphic Contrast value |
| Int | VCOMPO_SetVideoHue | set Graphic Hue value |
| Int | VCOMPO_GetVideoHue | Get Graphic Hue value |
| Int | VCOMPO_SetVideoSaturation | Set Graphic Saturation Value |
| Int | VCOMPO_GetVideoSaturation | Get Graphic Saturation value |

4. SOC DISPLAY API

| Return | Function | Description |
| --- | --- | --- |
| Return | Function | Description |
| Int | DISPLAY_Create | creates DISPLAY module and does initial works for this DISPLAY module |
| Int | DISPLAY_Destroy | destroys the DISPLAY module |
| Int | DISPLAY_GetSupportedResolutionList | gets DISPLAY GetSupported Resolution list |
| int | DISPLAY_GetResolution | Get video output Standard resolution |
| Int | DISPLAY_SetResolution | Set video output standard resolution |
| Int | DISPLAY_CEC_GetState | Get HDMI CEC status |
| Int | DISPLAY_CEC_SetEnable | Set HDMI CEC on/off |
| int | DISPLAY_CEC_SetPowerOnOff | Command CEC Power On/Off |
| Int | DISPLAY_HDR_GetState | Get hdr mode |
| int | DISPLAY_HDR_SetEnable | Set hdr mode |
| int | DISPLAY_HDR_GetSupported | Get hdr tv support state |
| Int | DISPLAY_HDMI_SetEnable | sets HDMI on/off |
| Int | DISPLAY_GetHdmiEdidInfo | gets DISPLAY GetHdmiEdidInfo |
| int | DISPLAY_SetHdmiHotpluggedListener | set HdmiHotplugged callback |
| int | DISPLAY_SetCECPowerStateListener | set DISPLAY_CECPowerState callback |

4.1  SOC DISPLAY Callback API

| callback | function | descriptor |
| --- | --- | --- |
| callback | function | descriptor |
| DISPLAY_HDMIHotPluggedCallback | typedef void (*DISPLAY_HDMIHotPluggedCallback)(int event) | Funtiont pointer for for HDMI Hot Plugged event callback |
| DISPLAY_CECPowerStateCallback | typedef void (*DISPLAY_CECPowerStateCallback)(int event) | Function pointer for CEC Power Stae event callback |

* OEM H/W Control HAL API

| Return | Function | Description |
| --- | --- | --- |
| Return | Function | Description |
| int | DEVICE_Create | creates DISPLAY module and does initial works for this DISPLAY module |
| int | DEVICE_Destroy | destroys the DISPLAY module |
| int | DEVICE_setLedState | Set front panel led state |
| int | DEVICE_setLedColor | Set front panel led color |
| int | DEVICE_setVolumeLedState | Set front panel led state for Volume |
| int | DEVICE_setBeamformingLedState | Set front panel led state for Beamforming |
| int | DEVICE_setMicOnOff | Set Mic on/off |
| int | DEVICE_getMicVolume | Get Mic volume level |
| int | DEVICE_SetMicVolume | Set Mic volume level |
| int | DEVICE_getMicEchoLevel | Get Mic echo level |
| int | DEVICE_setMicEchoLevel | Set Mic echo level |